Install Instructions for ICAS Multi-Fax Driver

1.	If you’re already using the PureData Fax driver, REMOVE this line from your CONFIG.SYS file:

DEVICE=C:\FAX\SATISFAX.SYS  IO=xxxx

2.	If you’re already using the PureData Fax driver, REMOVE this line from your AUTOEXEC.BAT file:

C:\FAX\CASMGR.EXE C:\FAX\CASMGR.CFG

If any of the following are present, remove them:

C:\FAX\FAXTSR.EXE          or 	C:\FAX\FAXPOP.EXE

3.	Reboot your computer in and stay in DOS, or if you’re running Windows 95, boot to the Safe-Mode Command Prompt Only by immediately pressing keyboard function key F8 several times when the computer says in text on-screen, “Starting Windows 95” at boot up.  You may have to try several time to get Windows 95 to boot this way, but you must be in the “Safe-Mode Command Prompt Only” mode to install the driver successfully!

4.        Insert the ICAS driver floppy disk into drive A: or drive B: and type A: ( or B: ) and hit the ENTER key

5.	Type INSTALL and hit the ENTER key

6.	After the files are installed, the install batch file will automatically run the CONFIG program which will be able to detect the fax cards.  If a fax card is shown when this program is run for the first time, hit ALT-R to remove any entries.

7.	Next, hit Alt-N ( For New Device ) and then choose which type of fax card you’re installing ( i.e. 200/400 ) using the keyboard arrows and hit the ENTER key.  The system will scan all possible addresses for the fax board and you should see a card detected at the address where it says “Board found.”  Next, hit a key to continue.

8.	Now, Press Alt-S on the keyboard to save the configuration set by CONFIG.

9.	If, in the future, you add or remove faxmodems, you must run CONFIG.EXE again to update your fax configuration file which is called C:\ICAS\BIN\I3LOAD.INI.

10.	If your faxmodem is going to be on the same phone line as your voice card, you will want to change the RINGS_BEFORE_ANSWER = 1 parameter in the I3LOAD.INI file to RINGS_BEFORE_ANSWER = 0 so that the fax card does not answer calls!

11.	ADD the following lines to your AUTOEXEC.BAT file:

cd \icas\bin

i3load

cd \